Supplier Quality Audit Checklist

(ISO 9001  ·  IATF 16949  ·  VDA 6.3)

A structured process-audit tool for evaluating supplier quality management systems against Tier-1/2 automotive requirements.

Built for Tier-1/2 automotive quality engineers

Version 1.0  ·  June 2026  ·  © Qualiteh Academy

qualiteh.com  ·  rankovic@qualiteh.com

Qualiteh Academy  ·  qualiteh.com  ·  rankovic@qualiteh.com  ·  Structured per ISO 9001 / IATF 16949 / VDA 6.3 methodology  ·  v1.0

Audit Details

| Supplier | Premier Stamping GmbH |
| --- | --- |
| Supplier code | SC-7890 |
| Site / plant | Plant 2, Stuttgart, Germany |
| Commodity / part family | Stamped structural body components |
| Audit type | Periodic Audit ☐ Initial ☑ Periodic ☐ Follow-up ☐ For-cause |
| Audit date(s) | 2026-06-15 |
| Lead auditor | A. Becker |
| Auditee contact | M. Weber, Quality Manager |
| Scope | ISO 9001:2015 / IATF 16949:2016 QMS — Stamping & Assembly (Cells 1–4) |

Scoring Guide

Per-Question Score Scale:

| Score | Status | Description |
| --- | --- | --- |
| 10 | Fully Met | Fully met — requirement completely fulfilled; objective evidence available |
| 8 | Largely Met | Largely met — minor improvement potential; no significant risk |
| 4 | Partially Met | Partially met — weak point identified; corrective action required |
| 0 | Not Met | Not met — requirement not fulfilled; major defect / business risk |

Degree of Fulfilment (DoF):

DoF % = (Sum of achieved scores) / (Maximum possible score) × 100

★ Critical questions (marked with ★):

A score of 0 on any ★-marked question automatically caps that element's effective DoF at 80%, regardless of all other question scores. This reflects a safety or systemic risk that cannot be offset by strength elsewhere.

Overall Audit Rating:

| Rating | DoF Range | Decision | Implication |
| --- | --- | --- | --- |
| A | ≥ 90% | Approved | Capable supplier — no critical action required |
| B | 80–89% | Conditional | Conditionally capable — improvement plan required within 60 days |
| C | < 80% | Hold | Not capable — business hold; re-audit required after corrective actions |

Note: A VDA 6.3 process audit (process element level) is methodologically distinct from an IATF 16949 QMS audit (system level). This checklist applies the 10/8/4/0 scoring convention of VDA 6.3 to both system-level and process-level questions for unified scoring across a supplier visit.

P1 — Quality Management System

| No. | Audit Question | Score (0/4/8/10) | Evidence / Finding | Action / Owner |
| --- | --- | --- | --- | --- |
| P1.1 | Is the scope of the QMS documented and aligned with the applicable standard (ISO 9001 / IATF 16949)? | 10 | QMS scope document Rev. 4 sighted; IATF 16949:2016 certificate current. | — |
| P1.2 | Are documented-information controls in place (creation, review, approval, version management, obsolescence)? | 8 | Document control procedure DP-001 in place. Minor gap: obsolete forms not consistently removed from shop floor. | Update obsolete-form retrieval process by 2026-07-31. Owner: QA Mgr |
| P1.3 | Is an internal audit programme established, implemented, and consistently executed at planned intervals? | 8 | Annual programme in place. One audit overdue by 6 weeks (Shipping). Evidence of corrective action initiated. | Close overdue audit by 2026-07-15. Owner: QA Mgr |
| P1.4 | Are management review records current and do they address required inputs/outputs per IATF 16949 clause 9.3? | 10 | Last management review 2026-05; minutes cover all required inputs including customer satisfaction and QMS performance trends. | — |
| P1.5 | Are key quality KPIs (PPM, scrap, warranty, on-time delivery) tracked, trended, and actioned at appropriate intervals? | 8 | KPI dashboard displayed; PPM trending upward for 3 months without documented root-cause escalation. | Initiate formal 8D on PPM trend by 2026-07-01. Owner: Quality Engineer |

P2 — Management & Planning

| No. | Audit Question | Score (0/4/8/10) | Evidence / Finding | Action / Owner |
| --- | --- | --- | --- | --- |
| P2.1 | Is a quality policy established, communicated, understood at all levels, and reviewed for continued suitability? | 10 | Quality policy posted in production and offices; awareness spot-check of 3 operators confirmed understanding. | — |
| P2.2 | Are quality objectives established, measurable, and linked to the strategic plan and customer requirements? | 8 | Objectives documented; not all are time-bound or have owner assignment at departmental level. | Add SMART attributes and owners to all Q2 objectives by 2026-07-31. Owner: Quality Director |
| P2.3 | Is APQP/product-launch project management in place with defined phase-gate milestones and cross-functional team participation? | 10 | APQP tracker sighted for active project SC-7890 Rev. A; cross-functional team roster and sign-offs confirmed. | — |
| P2.4 | ★ ★ Is a systematic risk management process (FMEA, risk register, contingency planning) in place and actively maintained? | 0 | Risk register last updated 2025-11. No evidence of annual review or update following a supplier change in Q1 2026. | CRITICAL: Update risk register immediately and establish quarterly review cadence. Owner: Plant Manager Due: 2026-07-08 |
| P2.5 | Are competence requirements defined, and is there evidence of training effectiveness evaluation for quality-critical roles? | 8 | Training matrix up to date. Effectiveness checks for welding operators not documented for 2026. | Document effectiveness evaluations for all Q-critical operators by 2026-08-31. Owner: HR + QA |

P3 — Product & Process Development

| No. | Audit Question | Score (0/4/8/10) | Evidence / Finding | Action / Owner |
| --- | --- | --- | --- | --- |
| P3.1 | ★ ★ Are DFMEA and PFMEA current, reviewed at defined intervals, and updated after design/process changes? | 8 | PFMEA Rev. 6 sighted; last updated 2026-04. Minor gap: one new process step added 2026-05 not yet reflected. | Update PFMEA for new step by 2026-07-15. Owner: Process Engineer |
| P3.2 | Is a control plan (pre-launch and production) in place covering all special characteristics? | 10 | Control Plan Rev. 5 current; all Sc/Cc characteristics identified and referenced from PFMEA. | — |
| P3.3 | ★ ★ Are PFD, PFMEA, and Control Plan aligned (tri-document alignment verified)? | 10 | Tri-document alignment matrix reviewed; all failure modes in PFMEA traceable to control actions in CP and process steps in PFD. | — |
| P3.4 | Are special characteristics (Sc/Cc/KPC) flowed down from customer and controlled at the point of manufacture? | 10 | Customer-specific requirements reviewed; Sc marked on drawings, control plan, and work instructions. | — |
| P3.5 | Is design validation / prototype evidence available and approved for new product introductions? | 8 | DVP&R available for active program; one test record missing customer approval sign-off. | Obtain customer approval on DVP&R by 2026-07-22. Owner: Program Manager |

P4 — Supplier / Sub-tier Management

| No. | Audit Question | Score (0/4/8/10) | Evidence / Finding | Action / Owner |
| --- | --- | --- | --- | --- |
| P4.1 | Is a process in place for selection, approval, and periodic re-evaluation of sub-tier suppliers? | 10 | Approved Supplier List (ASL) maintained; annual re-evaluation process documented and executed. | — |
| P4.2 | Is incoming inspection defined and commensurate with sub-supplier risk/performance? | 8 | Incoming inspection procedure in place. High-risk supplier sample plan not updated after supplier Q-incident in Q4 2025. | Update incoming inspection plan for affected supplier by 2026-07-15. Owner: Incoming QC Lead |
| P4.3 | ★ ★ Is material traceability maintained from sub-supplier through production and shipping (lot / heat / batch)? | 10 | Full lot traceability verified via ERP trace exercise on 3 finished parts; heat numbers traceable to mill cert. | — |
| P4.4 | Are customer-specific requirements (CSRs) flowed down to sub-tier suppliers in purchasing documents? | 8 | CSR flow-down clause in PO template; verification that sub-suppliers acknowledged CSRs not evidenced for 2 sub-suppliers. | Obtain CSR acknowledgements from all sub-suppliers by 2026-08-15. Owner: Purchasing |

P5 — Production & Process Control

| No. | Audit Question | Score (0/4/8/10) | Evidence / Finding | Action / Owner |
| --- | --- | --- | --- | --- |
| P5.1 | Are current, approved work instructions available at the point of use and understood by operators? | 10 | Work instructions (WI) Rev-controlled; WI sighted at each station on audit walkthrough; operator demonstrated understanding. | — |
| P5.2 | ★ ★ Are error-proofing (poka-yoke) devices verified at start-of-shift and after tool change; records maintained? | 8 | Poka-yoke verification sheets in place. Two devices showed verification gap on morning shift (not signed off). | Investigate and retrain shift leads on poka-yoke check discipline by 2026-07-10. Owner: Production Supervisor |
| P5.3 | Is SPC deployed at special-characteristic operations and are control charts acted upon using reaction plans? | 10 | SPC charts current on critical dimension; out-of-control reaction plans referenced and signed off by operators. | — |
| P5.4 | Are reaction plans defined for out-of-control and non-conforming product, and are they followed in practice? | 10 | Reaction plans integrated into WI; evidence of last non-conformance handled per plan (NCR #2026-041). | — |
| P5.5 | Is 5S systematically implemented and sustained with regular auditing? | 8 | 5S system in place; lay-by area near press line not meeting standard; most recent 5S audit score 78%. | Address lay-by area non-conformance and target 5S score ≥ 90% by 2026-08-01. Owner: Production Lead |
| P5.6 | ★ ★ Is part / material traceability maintained throughout production (job traveller, serialisation, labelling)? | 10 | Job travellers verified on production floor; part labels scanned at each operation; ERP records match physical. | — |

P6 — Measurement, Test & Calibration

| No. | Audit Question | Score (0/4/8/10) | Evidence / Finding | Action / Owner |
| --- | --- | --- | --- | --- |
| P6.1 | Are all measuring and test equipment identified, calibrated at defined intervals, and labelled with status? | 10 | Calibration database reviewed; all gauges current; colour-coded calibration stickers sighted on all instruments. | — |
| P6.2 | Have Measurement System Analyses (MSA / GR&R) been conducted for all special-characteristic measurement systems? | 8 | MSA studies on file for critical gauges; one new CMM fixture in use since 2026-03 without completed GR&R. | Complete GR&R for new CMM fixture by 2026-07-31. Owner: Metrology Lead |
| P6.3 | Is the lab scope defined, and is there evidence of accreditation or approved alternative where required by customers? | 10 | Internal lab scope documented; accredited external lab used for dimensional reports requiring customer submission. | — |
| P6.4 | Are retained samples / first-off / last-off parts maintained and traceable per customer and internal requirements? | 8 | Retained sample procedure in place; 2 of 15 part numbers checked had retention period expired without renewal decision. | Review and renew all retention decisions; update sample log by 2026-08-15. Owner: QA Inspector |

P7 — Nonconformance, 8D & Continuous Improvement

| No. | Audit Question | Score (0/4/8/10) | Evidence / Finding | Action / Owner |
| --- | --- | --- | --- | --- |
| P7.1 | Is a documented process for identifying, segregating, recording, and dispositioning nonconforming product in place? | 10 | NCR procedure documented; quarantine area clearly marked; last 5 NCRs dispositioned within SLA. | — |
| P7.2 | ★ ★ Does the 8D / problem-solving process demonstrate root-cause discipline (Is / Is-Not, Fishbone / 5-Why, D4 verified)? | 4 | 8D records reviewed; D4 root-cause analysis on 3 of last 5 customer complaints is superficial (symptom stated as root cause). D5 containment not always in place before D6. | Mandatory 8D retraining for all Quality Engineers by 2026-08-01. Coach next 3 8Ds live. Owner: Quality Manager |
| P7.3 | Is corrective-action effectiveness verified at defined intervals with objective evidence? | 8 | Effectiveness verification step in 8D procedure; 2 of 5 closed 8Ds lack effectiveness check sign-off at the 90-day mark. | Implement 90-day effectiveness check as a system control in QMS; audit 3 open actions by 2026-07-31. Owner: QA Mgr |
| P7.4 | Are customer complaint / warranty data systematically analysed and used to drive product / process improvements? | 10 | Warranty data reviewed monthly; Pareto analysis drives top-3 improvement projects; evidence in management review minutes. | — |
| P7.5 | Is a lessons-learned / read-across process in place to prevent recurrence on similar products / processes? | 8 | Lessons-learned log maintained in QMS; read-across not consistently executed across all product families. | Define read-across trigger criteria and execute for last 3 customer escapes by 2026-08-15. Owner: Senior Quality Eng |

Scoring Summary

| Element | Max Pts | Achieved | DoF % | ★ 0 Cap? | Eff. DoF % / Rating |
| --- | --- | --- | --- | --- | --- |
| P1 Quality Management System | 50 | 44 | 88.0% | No | 88.0% |
| P2 Management & Planning | 50 | 36 | 72.0% | YES — capped 80% | 72.0% (★ cap) |
| P3 Product & Process Development | 50 | 46 | 92.0% | No | 92.0% |
| P4 Supplier / Sub-tier Management | 40 | 36 | 90.0% | No | 90.0% |
| P5 Production & Process Control | 60 | 56 | 93.3% | No | 93.3% |
| P6 Measurement, Test & Calibration | 40 | 36 | 90.0% | No | 90.0% |
| P7 Nonconformance, 8D & Continuous Improvement | 50 | 40 | 80.0% | No | 80.0% |
| OVERALL — All Elements | 340 | 294 | 86.5% | — | 86.5% → Rating: B |

Overall Effective DoF: 86.5%  →  Rating B (Conditionally Capable — improvement plan required)

Note: Section P2 scored 0 on the ★-critical risk management question (P2.4), capping P2 effective DoF at 80% per the starred-question rule. Section P7 scored 4 on root-cause discipline (P7.2), resulting in a low element DoF. These two weaknesses drive the overall B rating and require immediate corrective action.

Findings & Corrective-Action Log

| # | Finding | Severity | Element | Corrective Action | Owner | Due Date | Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F-01 | Risk register last updated Nov 2025; not reviewed after supplier change Q1 2026. No evidence of annual review meeting. | Critical | P2 — Management & Planning | Update risk register immediately; establish quarterly review cadence; assign risk owner. | Plant Manager | 2026-07-08 | Open |
| F-02 | 8D root-cause analysis superficial on 3 of last 5 customer complaints (symptom stated as cause; D5 containment incomplete before D6). | Major | P7 — Nonconformance & CI | Mandatory 8D retraining for all QEs; live coaching on next 3 open 8Ds. | Quality Manager | 2026-08-01 | Open |
| F-03 | Error-proofing verification sheets not signed off on two poka-yoke devices on the morning shift (2026-06-15). | Minor | P5 — Production & Process Control | Retrain shift leads; add poka-yoke check to shift handover checklist. | Production Supervisor | 2026-07-10 | Open |

Sign-Off

| Role | Name / Title | Signature | Date |
| --- | --- | --- | --- |
| Lead Auditor | A. Becker | __________________ | 2026-06-15 |
| Auditee Representative | M. Weber, Quality Manager | __________________ | 2026-06-15 |

Distribution list:

Lead Auditor  ·  Quality Manager (Supplier)  ·  Supplier Development Manager  ·  Programme Manager  ·  Quality Director

| Re-audit required? | YES — targeted follow-up on P2 (risk) and P7 (8D) · Target date: 2026-09-01 |
| --- | --- |

Qualiteh Academy  ·  qualiteh.com  ·  rankovic@qualiteh.com  ·  Structured per ISO 9001 / IATF 16949 / VDA 6.3 methodology  ·  v1.0

Built by Qualiteh · Qualiteh OS Product Suite

This checklist is one output of the Qualiteh OS methodology — a suite of purpose-built tools for automotive quality engineers:

Qualiteh Audit:  Offline-first VDA 6.3 / LPA / custom-standard auditing with AEI-powered scoring. This checklist, digitized — run full VDA process audits on the shop floor, capture photographic evidence, generate 8D-ready findings, and track corrective actions to closure. Works without an internet connection.

Lens (PPAP Assistant):  Structured PPAP submission management from PPAP Level 1 through Level 5. Tracks part submission warrants, dimensional results, material certs, and PFMEA/CP alignment across multiple customers and programmes.

Sync (8D / A3):  Disciplined 8D and A3 problem-solving with structured root-cause guidance. Keeps containment, root-cause, and corrective-action records in one place — with read-across linking to prevent recurrence.

KPI Sentinel:  Real-time quality KPI tracking and trend alerting. PPM, scrap, warranty, OTD, and custom metrics with configurable thresholds and escalation paths.

Tacit CRM:  Relationship and opportunity management built for supplier development and quality account management — not a generic CRM adapted for manufacturing.

Run audits at the speed of AEI — qualiteh.com

Hands-on quality workshops — qualiteh.com/academy

AEI — Augmented Engineering Intelligence — is Qualiteh's term for the structured, context-aware guidance layer embedded in our products; a working aid that supports, not replaces, engineering judgement.

Engineers, not consultants.

Qualiteh Academy  ·  qualiteh.com  ·  rankovic@qualiteh.com  ·  Structured per ISO 9001 / IATF 16949 / VDA 6.3 methodology  ·  v1.0